# Kitchen Layout Specification

Martin Hinshelwood

January 14, 2026

Table of Contents

# Tall Cabinetry Wall

Tall Cabinetry Wall Render

Tall Cabinetry Wall Render

## Tall Cabinetry Wall – General Instructions

### Dimensions

- Height: Floor to ceiling

- Depth: Match adjacent tall cabinetry

### Materials and finish

- Solid Pirota wood, dark natural tone

- Flat, modern, handleless fronts

- Consistent reveals and shadow gaps across all tall cabinetry

### Hardware and environment

- Soft-close hardware throughout

- Hinges and runners suitable for humid tropical conditions

### Services

- Service voids only where required by appliance specifications or electrical code

- Electrical and ventilation to follow manufacturer requirements

- All electrical work to comply with Mexican codes

### Visual intent

- Tall cabinetry to read as a single continuous wall

- No visible filler panels on the front face

## 600 mm Tall Storage Section

### Dimensions

- Overall width: 600 mm

- Height: Floor to ceiling

- Depth: Match adjacent tall cabinetry

### Configuration

- Lower section to consist of deep drawers from floor level upward

- At a practical ergonomic height, drawers transition into a full-height cabinet

- Upper cabinet continues uninterrupted to the ceiling

### Drawer specification

- Drawers to be full-extension, deep format for general storage

- Solid Pirota wood fronts, handleless

- Heavy-duty soft-close runners

### Upper cabinet specification

- Single full-height door or split doors as required by height and hardware limits

- Door(s) to be handleless, flat modern profile

- Use soft-close hinges suitable for humid environments

- Internal layout configurable for pantry-style storage (adjustable shelves)

### Visual intent

- Section must read as a clean vertical volume

- Clean transition from drawers to cabinet with no visual breaks

## 800 mm Oven & Microwave Tower

### Dimensions

- Overall width: 800 mm

- Height: Floor to ceiling

- Depth: Match adjacent tall cabinetry

### Appliance configuration

- Oven: Integrated electric oven, installed at standard ergonomic oven height

  - Power: 220 V

  - Circuit: Dedicated

- Microwave: Freestanding microwave, visually integrated, installed directly above the oven at head height

  - Power: 110 V

  - Supply: May be shared

Appliances must be vertically aligned and centred within the cabinet

### Microwave integration

- Microwave to be installed using a recessed shadow-box opening

- Maintain a uniform 5–10 mm reveal on all sides

- Respect all manufacturer ventilation clearances

- Microwave must be fully removable without dismantling cabinetry

- No panels, doors, or cladding applied to the microwave

- Trim kits only to be used if explicitly supported by the selected model

### Lower section

- Drawers from floor level up to oven base

- Drawer fronts in solid Pirota wood, handleless

- Heavy-duty soft-close drawer runners

### Upper section

- Full-height cabinet above the microwave, extending to the ceiling

- Use vertical lift-up door where door height allows safe operation

- If door is too tall, split into two vertically aligned doors

- Soft-open and soft-close hardware suitable for humid environments

### Electrical

- 220 V circuit dedicated to the oven

- 110 V supply for microwave may be shared

- Outlets to be accessible without dismantling cabinetry

- Circuits to be clearly labelled in the electrical panel

### Services and safety

- Service voids only where required by appliance or electrical code

- Heat shielding where required by oven manufacturer

### Visual intent

- Oven and microwave to read as a single integrated vertical composition

- Section width to visually match the 800 mm appliance storage section

## 908 mm Refrigerator Section

### Dimensions

- Overall width: 908 mm

- Height: Floor to ceiling

- Depth: Match adjacent tall cabinetry

### Appliance configuration

- Refrigerator: Full-height French-door refrigerator with bottom freezer drawer

- Appliance installed as freestanding but visually integrated

- Refrigerator centred within the opening

- Manufacturer-specified clearances maintained

### Upper cabinet

- Cabinet above refrigerator extending fully to the ceiling

- Door to be handleless, flat modern profile

- Soft-close hinges suitable for humid environments

- Split doors if required by height and hardware limits

### Ventilation and services

- Ventilation per refrigerator manufacturer requirements

- Electrical outlet 110 V, shared supply acceptable

- Outlet to remain accessible

- Circuit to be labelled in the electrical panel

### Visual intent

- Refrigerator to read as a primary integrated appliance

- Upper cabinet completes the vertical rhythm of the wall

## 800 mm Appliance Storage (Appliance Garage)

### Dimensions

- Overall width: 800 mm

- Height: Floor to ceiling

- Depth: Match adjacent tall cabinetry

### Lower section

- Drawers from floor level up to standard counter height

- Deep-format drawers for small appliance and general storage

- Drawer fronts in solid Pirota wood, handleless

- Heavy-duty soft-close drawer runners

- Drawer heights aligned visually with adjacent base cabinetry

### Upper section

- Full-height appliance storage behind doors, from counter height to ceiling

- Intended for coffee station and small appliances

- Adjustable internal shelving sized for coffee machine, grinder, and accessories

- Integrated warm LED lighting

### Doors

- Side-hinged bi-fold doors, two leaves per side

- Doors open outward and fold flat against the cabinet side panels

- Doors park fully open, held with magnetic or mechanical stays

- Soft-close hinges suitable for humid environments

- No pocket doors, no roller systems, no lift-up mechanisms

- Handleless, flat profile

- Grain direction matched across doors and aligned with adjacent cabinetry

### Electrical

- Internal 110 V outlets for appliances

- Shared supply acceptable

- Outlets concealed but accessible

- Wiring rated for humid tropical conditions

### Services and ventilation

- Ventilation appropriate for coffee machines and small appliances

- Service voids only where required

- No active exhaust unless later specified

### Visual intent

- Closed: reads as a single uninterrupted cabinet volume

- Open: doors do not obstruct access or project into circulation space

- Width balanced with the 800 mm oven/microwave tower

Confidence rating: 0.98

# Continuous Base Cabinetry

Base Cabinetry Render

Base Cabinetry Render

## Overview

This section spans two walls and provides continuous base cabinetry with integrated sink, dishwasher, and storage.

### Dimensions

- Height: Standard counter height (TBD)

- Depth: Standard base cabinet depth (TBD)

- Total run: TBD mm (spanning two walls)

### Materials and finish

- Solid Pirota wood, dark natural tone

- Flat, modern, handleless fronts

- Consistent with tall cabinetry aesthetic

### Hardware

- Soft-close hardware throughout

- Hinges and runners suitable for humid tropical conditions

## Subsections

To be documented: - Sink section with specifications - Dishwasher integration - Base storage cabinets - Countertop specifications - Backsplash details - Electrical outlets - Plumbing rough-in locations

# Kitchen Island

Island Render

Island Render

## Overview

Freestanding kitchen island providing additional workspace, storage, and seating.

### Dimensions

- Length: TBD mm

- Width: TBD mm

- Height: Standard counter height (TBD)

### Materials and finish

- Solid Pirota wood, dark natural tone

- Flat, modern, handleless fronts

- Consistent with overall kitchen aesthetic

### Configuration

- Storage drawers and/or cabinets

- Countertop overhang for seating (if applicable)

- Electrical outlets

- Plumbing (if required)

## Subsections

To be documented: - Storage configuration - Countertop specifications - Seating details (if applicable) - Electrical requirements - Plumbing requirements (if applicable)

## Materials Specification

### Cabinet construction

- Carcases:

  - Madera (engineered wood board), suitable for cabinetry.

  - All panels sealed on all faces and edges, including backs.

- Fronts:

  - Parota veneer–covered fronts (Parota/Pirota appearance).

  - Flat, modern, handleless design.

### Finish and treatment

- Colour: Dark natural Parota tone, grain visible.

- Finish system:

  - Fully sealed and suitable for humid tropical and coastal conditions (Cancún).

  - Moisture-resistant and mould-resistant.

- Edge treatment:

  - All edges sealed, no exposed substrate.

### Countertops

- Existing granite worktop retained.

- Only localised cutting or adjustment permitted where specified (hob infill).

- No replacement or refinishing assumed.

### Hardware

- Drawer runners:

  - Heavy-duty, full-extension, soft-close.

  - Corrosion-resistant.

- Hinges:

  - Soft-close.

  - Rated for humid and coastal environments.

- Handles:

  - None.

  - Handleless achieved through reveals, shadow gaps, or finger pulls.

### Sealants and adhesives

- All glues, sealants, and bonding agents to be:

  - Rated for humid tropical climates.

  - Resistant to delamination.

  - Food-safe where applicable.

### General build requirements

- Allow for seasonal movement between carcass and veneered fronts.

- No unsealed end grain anywhere in the build.

- All fixings to be corrosion-resistant.

- Workmanship suitable for long-term use in a coastal Cancún environment.

# Electrical Specification

All electrical work to comply with Mexican electrical codes.

Project Scope: This is primarily a cabinet and appliance upgrade with limited ability to add new electrical circuits. Most appliances will use existing electrical infrastructure.

## NEW CIRCUITS

### 220V Oven Circuit (NEW - REQUIRED)

- Location: Tall Cabinetry Wall - 800mm Oven Tower

- Load: Electric oven (see appliances-list.md)

- Requirements:

  - Dedicated 220V circuit

  - Accessible outlet without cabinet removal

  - Must be installed before cabinetry

  - Circuit to be clearly labeled in electrical panel

## EXISTING 110V CIRCUITS (TO BE UTILIZED)

### Refrigerator

- Location: Tall Cabinetry Wall - 908mm Refrigerator Section

- Load: French-door refrigerator

- Requirements: Use existing outlet, verify accessibility after cabinet installation

### Microwave

- Location: Tall Cabinetry Wall - 800mm Oven Tower

- Load: Freestanding microwave

- Requirements: Use existing outlet or share existing circuit, verify accessibility

### Appliance Storage (Coffee Station)

- Location: Tall Cabinetry Wall - 800mm Appliance Garage

- Load: Coffee machine, grinder, small appliances

- Requirements:

  - Utilize existing outlets

  - Internal outlets concealed but accessible

  - Verify existing circuit capacity for multiple appliances

### Base Cabinetry

- Location: Continuous Base Cabinetry

- Load: Task lighting, small appliances, dishwasher

- Requirements:

  - Utilize existing outlets along backsplash

  - Verify outlet locations work with new cabinet layout

  - Relocate outlet boxes if required by cabinet placement (no new circuits)

### Island

- Location: Kitchen Island

- Status: No electrical circuits available

- Note: Island will not have electrical outlets

## Pre-Installation Requirements

- Survey existing outlets - Map all current outlet locations and circuit capacities

- Verify oven circuit installation - 220V circuit must be complete before cabinet installation

- Test existing circuits - Confirm all circuits to be used are functioning and properly grounded

- Plan outlet accessibility - Ensure all outlets remain accessible after cabinet installation

## Panel Labeling

All circuits (new and existing) to be clearly labeled in the electrical panel with location and purpose.

## Environmental Considerations

All wiring and connections rated for humid tropical conditions.

# Plumbing Specification

All plumbing work to comply with Mexican plumbing codes.

Project Scope: Existing plumbing locations will be utilized. New plumbing locations are not possible. Cabinet layout must accommodate existing plumbing infrastructure.

## Water Supply (EXISTING LOCATIONS)

### Sink

- Location: Continuous Base Cabinetry

- Type: To be specified

- Supply: Hot and cold water (existing connections)

- Requirements:

  - Use existing supply lines

  - Verify shut-off valves are functional and accessible

  - Cabinet layout must accommodate existing plumbing location

### Dishwasher

- Location: Continuous Base Cabinetry (adjacent to sink)

- Supply: Hot water connection (existing or shared with sink)

- Requirements:

  - Use existing connection if present

  - If no existing dishwasher connection, tap from sink supply

  - Shut-off valve accessible

### Refrigerator (if ice maker/water dispenser)

- Location: Tall Cabinetry Wall - 908mm Refrigerator Section

- Supply: Cold water line (if existing connection available)

- Requirements:

  - Only if existing water line is present in this location

  - If no existing line, refrigerator model without water/ice features

  - Shut-off valve accessible if used

### Island

- Location: Kitchen Island

- Status: No plumbing available

- Note: Island will not have sink or water features

## Drainage (EXISTING LOCATIONS)

### Sink

- Location: Continuous Base Cabinetry

- Requirements:

  - Use existing drain location

  - Verify P-trap and venting are functional

  - Cabinet layout must accommodate existing drain location

### Dishwasher

- Location: Continuous Base Cabinetry

- Requirements:

  - Drain connection to sink drain or existing dedicated drain line

  - Verify connection is functional

## Pre-Installation Requirements

- Survey existing plumbing - Map all current supply and drain locations

- Test all connections - Verify supply lines, shut-offs, and drains are functional

- Plan cabinet layout - Ensure cabinet design accommodates fixed plumbing locations

- Verify accessibility - All shut-off valves must remain accessible after cabinet installation

## Environmental Considerations

All materials and connections suitable for humid tropical conditions.

# Appliances List – Cabinetry & Installation Specification

## Major Appliances

## Refrigerator

Make: Samsung
Model: RF32CGSN10B1
Type: Full-height French-door refrigerator with bottom freezer drawer
Location: Tall Cabinetry Wall, 908 mm dedicated section
Installation type: Freestanding, visually integrated

### Dimensions (external)

- Width: 908 mm

- Height: 1,778 mm

- Depth: 861 mm

- Effective depth incl. door clearance: ~910 mm

### Installation requirements

- Electrical: 110 V, standard household outlet (adjacent cabinet)

- Water: Not required (no ice or water dispenser)

- Side clearance: 10–20 mm each side

- Top clearance: ≥25 mm

- Rear clearance: ≥25 mm (airflow and power cable)

- Floor: Level, capable of supporting fully loaded appliance

- Ventilation: Passive ventilation required if boxed in

### Cabinet maker instructions

- Provide a clear internal width of 908 mm minimum, excluding fillers.

- Do not clamp the refrigerator tightly between panels.

- If flanked by tall units, include:

  - Either a vented plinth below, or

  - A vent slot or shadow gap above.

- Allow full door swing of both French doors and freezer drawer.

- Side panels to stop short of door hinge plane to avoid interference.

- Appliance remains freestanding, no fixing to cabinetry.

## Oven

Type: Integrated electric oven
Model: To be selected
Location: Tall Cabinetry Wall, 800 mm oven/microwave tower
Installation height: Ergonomic (mid-level)

### Installation requirements

- Electrical: 220 V, dedicated circuit

- Ventilation: As per manufacturer

- Heat shielding: As specified by oven manufacturer

### Cabinet maker instructions

- Build tower to standard 600 mm oven module width unless specified otherwise.

- Provide manufacturer-specified ventilation gaps top and rear.

- Use heat-resistant cabinet lining where required.

- Oven to be fully supported on cabinet base, not rails alone.

- Final cut-outs and clearances to be confirmed once model is selected.

## Hob (Cooktop)

Brand: IO Mabe
Model: PIO90SMXN0
Series: Tecnología Ultimate
Type: Built-in gas hob, black tempered glass
Location: Continuous base cabinetry
Installation type: Drop-in

### Dimensions (overall)

- Width: 900 mm

- Depth: 510 mm

- Height: 130 mm

### Specifications

- Burners: 5

- Fuel: Gas

- Surface: Black tempered glass

- Controls: Front-mounted knobs

### Cut-out dimensions (estimated – must be verified)

- Width: 860–870 mm

- Depth: 490–500 mm

- Corner radius: R5–R10 mm

⚠️ Critical: IO Mabe does not publish official cut-out dimensions.
Measure the metal chassis under the glass before cutting granite.

### Installation requirements

- Gas supply to local code

- Front and underside ventilation as required

- Granite cutting required

- Black powder-coated aluminium infill at front edge

### Cabinet maker instructions

- Design base cabinet run assuming 90 cm hob width, drawers below.

- Coordinate with stone fabricator before cabinet finalisation.

- Allow for a full-width matte black powder-coated aluminium infill strip at the front of the hob cut-out to correct depth difference.

- Ensure cabinet rails do not obstruct hob body or gas connection.

- No oven below, drawers only.

- Do not rely on glass overhang for structural support.

## Microwave

Type: Freestanding microwave
Model: To be selected
Location: Tall Cabinetry Wall, above oven
Installation type: Recessed, removable

### Installation requirements

- Electrical: 110 V

- Ventilation: Per manufacturer

- Mounting: Shadow-box style

### Cabinet maker instructions

- Create a recessed microwave bay, not fixed installation.

- Provide 5–10 mm shadow reveal on all visible sides.

- Internal power outlet required inside the cabinet.

- Microwave must be fully removable without cabinet dismantling.

- Do not assume trim kit unless model is confirmed.

## Dishwasher

Make: Samsung
Model: DW80CG5420SREM
Type: Fully integrated built-in dishwasher
Location: Continuous base cabinetry
Finish: Stainless steel (panel-ready compatible)

### Dimensions

- Width: 598 mm

- Height: 845 mm (adjustable feet, approx. 835–870 mm range)

- Depth: 600 mm (excluding door panel)

### Cabinet opening required

- Width: 600 mm

- Height: 850 mm nominal

- Depth: ≥610 mm (hoses and power)

### Installation requirements

- Electrical: 120 V / 60 Hz, grounded outlet in adjacent cabinet

- Water: Cold water, ¾″ connection

- Drain: High loop or air gap as per local code

- Ventilation: Front venting via toe-kick

### Cabinet maker instructions

- Provide exact 600 mm clear opening, no fillers intruding.

- Door to accept custom Pirota wood panel aligned with adjacent drawers.

- Maintain clearance for AutoRelease™ door opening.

- Ensure toe-kick is removable and aligned with recessed black plinth.

- No side ventilation required.

## Small Appliances (Appliance Garage)

### Coffee Machine

- Location: 800 mm appliance storage cabinet

- Power: 110 V

- Requirements: Internal socket, ventilation

### Coffee Grinder

- Location: 800 mm appliance storage cabinet

- Power: 110 V

### Additional small appliances

- Toaster, blender, mixer, etc.

- Adjustable shelving required

### Cabinet maker instructions (appliance garage)

- Provide multiple internal outlets.

- Include warm LED task lighting.

- Allow rear ventilation gap.

- Shelves adjustable, load-rated for appliances.

- Doors to conceal appliances fully when closed.